# Line Following Robot

Frank Bruno	                                         James Ihrig

University of Central Florida                                  University of Central Florida
4000 Central Florida Blvd                                      4000 Central Florida Blvd

Orlando, Florida, 32816                                         Orlando, Florida, 32816

fbruno13@knights.ucf.edu                                          JEIhrig@gmail.com

Abstract

The abstract will talk about the project as a whole in no more than 150 words.

# Introduction

A line following robot is a simple robot design that uses sensors to make small adjustments to the motors in order to follow a line on an opposite contrast surface.  This line following robot happens to read a black line on a white surface, as seen in Figure 1.  This line following robot aims to achieve a high speed, low error, and low cost solution to following any line laid out by a user.

Figure 1 – The line following robot and the track it will be running on.

This line following robot makes use of a PID controller to follow the line using input from an array of IR led and photodiode pairs.  The design of the hardware was simplified to using many off the shelf components to allow for fast prototyping and a focus on the software related to a PID controller.  In addition, this project aims to introduce learning to the tedious process of manually tuning a PID controller.

The motivations for having this automatically tunable PID controller are many.  It would allow any speed motor to be trained without having to rely on manual methods that could take a very long time to learn in a supervised environment.  The automatic method that is purposed here is a semi-supervised method with the user having to lay out the track and specify a start position using a surface consisting of a black line that is long enough to hit all of the sensors.  The robot is designed to stop if it runs off the track at which point the user must manually place the robot back on the track.

# Related Work

This section talk about the work being done that relates to this project.  There are various methods of line following robots out there.  Many of them are the same, following the time old tradition of using IR led /photodiode pairs to act as the eyes of the robot.  A very recent paper submitted to the ICCAE done by Pakdaman et al. shows the design and implementation of a simple line following robot using just this method.  The benefits of using this design are it is simple, cost effective, and reliable.  Some have improved upon the reliability of the IR led/photodiode pair by using an IR emitter and an IR NPN phototransistor, which are very cheap and more reliable than the original pairing.  Others, like Dupuis and Parizeau, have successfully used cameras.  Each of these sensors has their own advantages and disadvantages.

Most work done on line following robots use a low cost, low power microcontroller to do the job of reading in values from sensors and controlling the motors.  Pakdaman et al. uses this method.  Another method used by some is a transistor method that doesn’t use microcontrollers at all.  This method relies on a hardware only approach to follow the line.  A disadvantage of this is has no method of learning if the implementers would like a better line follower.

Various control techniques have been used to control the robot to do line following.  The simplest of them is the PID controller.  This method uses a feedback control loop to adjust the turn of the wheels to follow the line.  For this method, manual tuning must occur to get a good line follower.  Other methods include Collins and Wyeth’s method of using a Cerebellar Network to learn the correct turn from the sensors and Dupuis and Parizeau’s method of genetic programming to evolve a control method.

# Method

This section focuses on the implementation of the line following robot.  It consists of the hardware design description, PID controller, and the PID learning algorithm.

Hardware

This section discusses the hardware used to make the line following robot.  The focus of this section is on the design decisions made while constructing the robot such as what parts were used in creation of the robot including sensors, microcontroller (MCU), Wireless Command and Reporting, Power, and Motors.  It will also cover some of the hardware limitations and the cost of the robot.

Chassis

Figure 2 - Pololu QTR-8 Sensor Array

As discussed in the related work, many line following robots are small RC car like robots.  They can have 2 or more wheels.  The smaller and lighter the robot the more speed the robot will have.  The body of the line following robot will consist of a Pololu 5" Robot Chassis RRC04A Solid Blue base with a Pololu 3pi Expansion Kit without Cutouts – Black mounted to it 1.5” above.  The expansion kit is essentially a prototyping board the same dimensions as the base.  This chassis allows for the most flexibility to design the rest of the robot and is very light weight.  Additionally, the base comes with cutouts for the wheels and many predrilled holes.  This design does limit the amount of wheels to 2, but given the light weight design 2 wheels should be sufficient.  Since the robot is using only 2 wheels, a Pololu Ball Caster with 3/8" Metal Ball will be used with a counter weight to keep the robot balanced.  The ball will just roll as it makes contact with the surface.

Sensors

As discussed in the related work, many line following robots simply use some form of an IR led and a Photodiode used to measure the reflectance from the surface directly below the robot.  This combination will output a voltage based on the level of reflectance from the surface.  This voltage will be close to zero on a white surface and close to the voltage of the input source on a black surface.  A very important choice that needs to be made is how many IR led and Photodiode pairs (sensors) will be used.  This is mainly dependent on the size of the robot and the width of the tape.  Ideally, the amount of sensors is as many as possible to spread across the width of the robot without the sensors interfering with each other.  Since the track is made using 0.75” inch electrical tape, see Figure 1, the ideal width is smaller than the tape.  This way more than one sensor would be able to see the tape at a time.  Also, the sensor array shouldn’t be much longer than the robot which is 5 inches.  Given these specifications, the ideal amount of sensors is 8 or at the worst 6.

With these specifications in mind, there are two easy choices for the sensor array the Pololu QTR-8A Reflectance Sensor Array, shown in Figure 2, and Pololu QTR-8RC Reflectance Sensor Array.  With a good price point of only 15 dollars each, both are very good choices.  The QTR-8 comes with 8 IR/Photodiode pairs and can be run at either 5 or 3.3 volts, very common voltage ratings.  The spacing between each sensor is 0.375 inches.  This falls in line with the required specifications.  The only difference between them is the RC can be hooked up to any digital I/O and the A must be hooked up to an analog input only.  Since either of them are good choices the choice of the MCU will determine, which sensor array will be chosen based on the number of analog inputs.

Motors

The motors are a very important since they will be the driving force propelling the robot forward.  An important goal of this project is to create a line following robot that is very fast.  Therefore, a variety of motors were researched from Pololu.  All of these motors have a voltage rating of 6 volts, but will work with any voltages between 3 and 9 volts. So, they are compatible with the already chosen sensor array.  Additionaly, these motors are small 0.94" x 0.39" x 0.47" and 0.35 oz. The first of which is a 50:1 Micro Metal Gearmotor HP with Extended Motor Shaft.  This motor is very fast with 625 RPM, they use 100 mA when free running and 1.6 A at stall.  This motor has 15 oz-in of torque. The second is a 50:1 Micro Metal Gearmotor with Extended Motor Shaft.  This motor is much slower than the previous with only 250 RPM, but they use just 40 mA when free running and 360 mA at stall. This motor has 6 oz-in of torque. The last  motor reviewed is a 100:1 Micro Metal Gearmotor with Extended Motor Shaft.  This motor is the slowest of the motors reviewed with 120 RPM, they use 40 mA when free running and 360 mA at stall.  This motor has 10 oz-in of torque. Although, they are the slowest they provide more torque than the 50:1 standard motors.  The ideal choice is the 50:1 HP motors because it meets our specifications better than the others, but it requires a power source that can provide up to 3.2 A if the wheels stall, and a MCU that can output at least 100mA and up to 1.6 A per motor.  Since Pololu offers brackets and wheels that will fit any of the motors reviewed, the wheels for the line following robot were chosen to be a pair Pololu Wheels which are 42x19mm and the pair of brackets chosen were the Pololu Micro Metal Gearmotor Bracket Extended.

Robot Controller

Figure 4 – Voltage Regulator

With the sensors and motors chosen, the next choice is what will receive the input from the sensors and use this data to drive our motors to achieve the goal of line following.  The Pololu Baby Orangutan B-328P was chosen as our robot controller, shown in Figure 3.  This decision was based on the fact that this robot controller uses an 8-bit Atmega328p microcontroller in a 24-pin form factor complete with headers for an easy to manage prototyping environment, which is ideal given that a prototyping board is being used. The Atmeg328p comes with the ability to drive two independent motors with a continuous current supply of 1 Amp per motor with a peak current at 3 Amps per motor.  This falls in line with any motor that was discussed.  Therefore, the 50:1 motors will be used since they provide the most stable prototyping environment with medium velocity.  In addition, this MCU comes with 8 analog inputs, which is exactly how many we need for the 8 sensor inputs.  Therefore, the QTR8-A sensor array will be used.  Also, the recommended input voltage is between 5 – 13.5 volts.

Power

In order to power these devices, a power supply must be chosen.  Additionally, the voltage must be regulated using a voltage regulator.  This insures no spikes from a battery will cripple any of the components.  Since all of the components have support for an input of 5 volts, 5 volts is used as the voltage for the robot.  The LT1963ET voltage regulator from linear technologies is used to do the job of regulating the voltage, the schematic is shown in Figure 2.  It can receive input voltages of ± 20 V and supply 1.5 A to the robot at any voltage from 1.21 V to 20 V.  Our ideal choice for a battery is a 9 V, but due to the large supply of current required from the motors a single 9 V cannot be used.  So, instead the robot uses a 12 V Thunder Power Rechargeable Li-Polymer battery with a capacity of 2070 mA/h.  This allows the robot to run for a few hours and a rechargeable battery saves buying new batteries.

Wireless Command and Reporting

Figure 3 - Pololu Baby Orangutan B-328P for full schematic see Appendix A. Figure 6

The wireless command and reporting feature is an optional feature that helped the debug process and created a more accurate evaluation this robot.  A pair of XBee-PRO 900 extended range modules were used to communicate between a laptop and the robot.  This allowed for on the fly debugging and the reporting of run data including the total error for that lap around the track.  This could also be used to send commands to the robot wirelessly.  Many of the uses of this feature will be discussed further in the evaluation section of this paper.  Unfortunately, these modules only run at 3.3 V and in order to use this module an AnyVolt Micro Universal DC-DC Converter is used to bring the output voltage down to 3.3 V for this part only.  Since this is an optional feature the cost of these parts will be deduced from the total cost to build, but the price will be include for reference.

Cost

The cost of each of the major components is listed in Table 1 and the optional components are listed in Table 2.

| Product | Cost | Quantity | Total Price |
| --- | --- | --- | --- |
| Pololu Wheel 42x19mm Pair | $6.98 | 1 | $6.98 |
| Pololu Micro Metal Gearmotor Bracket Extended Pair | $4.99 | 1 | $4.99 |
| Baby Orangutan B-328 + USB AVR Programmer Combo | $31.95 | 1 | $31.95 |
| Pololu 5" Robot Chassis RRC04A Solid Blue | $7.95 | 1 | $7.95 |
| Pololu Ball Caster with 3/8" Metal Ball | $2.99 | 1 | $2.99 |
| 3pi Expansion Kit without Cutouts - Black | $19.95 | 1 | $19.95 |
| LT1963ET | $3.00 | 1 | $3.00 |
| QTR-8A Reflectance Sensor Array | $14.95 | 1 | $14.95 |
| 50:1 Micro Metal Gearmotor with Extended Motor Shaft | $16.95 | 2 | $33.90 |
| Grand Total |  |  | $126.66 |

Table 1 – Bill of Materials

| Product | Cost | Quantity | Total Price |
| --- | --- | --- | --- |
| AnyVolt Micro Universal DC-DC Converter | $19.99 | 1 | $19.99 |
| XBee-PRO 900 extended range module w/ wire antenna | $39.00 | 1 | $39.00 |
| 50:1 Micro Metal Gearmotor HP with Extended Motor Shaft | $16.95 | 2 | $33.90 |
| 100:1 Micro Metal Gearmotor with Extended Motor Shaft | $16.95 | 2 | $33.90 |

Table 2 – Optional and substitute parts

PID controller

To control our robot, we have decided to use a well know method known as a PID controller. PID stands for Proportional Integral Derivative. A PID controller works based on current error, duration of error and change in error and tries to counter such error in a balanced way that results in a smooth accurate tracking. When deciding to use a PID controller, the first thing that must be decided is how to calculate the initial error. For our robot, our error is calculated by checking if each of our sensors is activated (above some threshold.). Specifically, it can be described as:

NUM_LINE_SENSORS = 8

DESIRED = 0

for ( i = 0; i < NUM_LINE_SENSORS; i++)

{

lineSensor[i] = analogRead(i)

if (lineSensor[i] > SENSOR_THRESHOLD)

{

if (i < NUM_LINE_SENSORS/2)

{

actual -=  (NUM_LINE_SENSORS/2 - i)

}

else

{

actual += (i - NUM_LINE_SENSORS/2 - 1)

}

}

}

error = DESIRED – actual

Where linesSensor[i] can range between [0, 1023], depending on activation level, and SENSOR_THRESHOLD is set to some value in the range [0, 1023], in our case it is 400. A simplified way to look at this is to check if each sensor is activated, if it is, then add to error if it's on the right side, subtract if it's on the left side.

Now that we have an error, we can start to construct a PID controller. The proportional component of the PID controller tells the robot to turn proportionally to the error., the integral component tells the robot to use the sum of error over time to determine how much to turn when current correction has not been enough to correct the error over time, and the derivative component tells the robot uses the rate of change of the error to dampen the turn rate to help prevent over-correction. We multiply each of these terms by some constant to tune each constant until we get good results. First we add a P value only and test it on the track, tweaking it until it seems to track with minimal oscillation. We then add a D term to dampen some of the oscillation. The I term is then added to ensure that the robot does not take turns too widely.

We have found some constants for P, I and D that yields some good results, but we will include some results from running other values near those that we found to show the differences that are seen when other values are used. In our tests so far, the best constants for P, I and D we have found are 50, 0.05, and 10 respectively.

This section should talk about the PID controller and how it works.  It should include Explanation of the controller and terms include pseudo-code and the control system loop. Explain how we are calculating error and how the manual tuning process works for us. As well as, the values of P, I, and D we used for our default values that yielded good results.

Here is the code we used for our PID controller:

P = 50, I = 0.05, D = 10

//The P multiplicand

error = desired - actual;

//The D multiplicand

dError = 2(error - oldError) / (currentTime() - lastFrameTime));

//The I multiplicand

sumError += error * (currentTime) - lastFrameTime);

//Set old error to current

oldError = error;

lastFrameTime = currentTime();

//Turn rate

return P * error + I * sumError + D * dError;

Learning PID values (0.5 – 1 page) J

This section should talk about the learning technique used to find PID values.

Error for each run will be calculated by using both the lap time and the sum of the absolute value of error over the entire run. We will score it with a bias toward speed over control by multiplying our speed to affect the score by 50% while allowing the error correction to account for 50%. The goal is the fastest lap-time possible, and any values of P, I, and D that cannot consistently keep the robot on the track will be thrown out.

# Evaluation (2 total pages)

This project is evaluated with respect to different values of P, I, and D and the performance score received for an average of 5 laps around the track.  The performance score is based on speed contributing 50% and the total accuracy being 50%.  So, the speed will take priority over the error, but still allow for more accurate values to overcome the speed.  Both the accuracy and the lap time are defined as a lower is better value. These values are recorded by the microcontroller and reported back to the laptop using the wireless serial communication module.

The first set of tests is done on values set from observable good runs done during the initial testing of this robot.  These values and the performance with respect to error rate and speed are shown in Table 3.   Additionally, the values are compared with the values from automated learning.

| Run | P | I | D | Error | Time | Score |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 40 | 0 | 0 | 4447 | 8.274 | 6361 |
| 2 | 40 | 0 | 10 | 4600.2 | 8.259 | 6430 |
| 3 | 40 | 0 | 15 | 4109 | 8.309 | 6209 |
| 4 | 40 | 0.05 | 0 | 3917 | 8.173 | 6045 |
| 5 | 40 | 0.05 | 10 | 3814.8 | 8.202 | 6008 |
| 6 | 40 | 0.05 | 15 | 4021.8 | 8.210 | 6116 |
| 7 | 40 | 0.1 | 0 | 3372.6 | 7.964 | 5668 |
| 8 | 40 | 0.1 | 10 | 3552 | 8.3 | 5926 |
| 9 | 40 | 0.1 | 15 | 3468.8 | 8.380 | 5924 |
| 10 | 50 | 0 | 0 | 3579.4 | 7.964 | 5772 |
| 11 | 50 | 0 | 10 | 3712 | 8.308 | 6010 |
| 12 | 50 | 0 | 15 | 3711 | 8.416 | 6064 |
| 13 | 50 | 0.05 | 0 | 3631.6 | 8.292 | 5962 |
| 14 | 50 | 0.05 | 10 | 3414.4 | 8.332 | 5873 |
| 15 | 50 | 0.05 | 15 | 3503 | 8.366 | 5935 |
| 16 | 50 | 0.1 | 0 | 3294 | 8.311 | 5803 |
| 17 | 50 | 0.1 | 10 | 3270.2 | 8.487 | 5879 |
| 18 | 50 | 0.1 | 15 | 3819.8 | 8.598 | 6209 |
| 19 | 60 | 0 | 0 | 3498.6 | 8.293 | 5896 |
| 20 | 60 | 0 | 10 | 3812.4 | 8.618 | 6215 |
| 21 | 60 | 0 | 15 | 3963.6 | 8.488 | 6226 |
| 22 | 60 | 0.05 | 0 | 3062 | 8.197 | 5630 |
| 23 | 60 | 0.05 | 10 | 3613.2 | 8.731 | 6172 |
| 24 | 60 | 0.05 | 15 | 3676 | 8.746 | 6211 |
| 25 | 60 | 0.1 | 0 | 3498.6 | 8.293 | 5896 |
| 26 | 60 | 0.1 | 10 | 3252.6 | 8.450 | 5851 |
| 27 | 60 | 0.1 | 15 | 3287.2 | 8.563 | 5925 |

Table 3 – Experiments with Different PID Values

Comparison between Controllers (1 page) F

A PID controller to a P, PI, and PD controller will be tested with similar values.  This will show the benefit to having each of the terms and how they affect the performance.

Comparison of PID Values (1 page) J

The PID values found in the method section will be tested against the learned PID values.  This will show the benefit of the learned PID values over the user set values.

# Conclusion (0.5 – 1 page) F J

This section should be concluding remarks about the project talking about the strengths, weaknesses, and possible improvements.

# References

Collins, D.; Wyeth, G.: Cerebellar control of a line following robot. In: Proceedings of the Australian conference on robotics and automation (ACRA ’99), p. 74–79. (1999)

Dupuis, J.-F., Parizeau, M.: Evolving a vision-based line-following robot controller. In: The 3rd Canadian Conference on Computer and Robot Vision (CRV 2006), p. 75. IEEE Computer Society Press, Los Alamitos (2006)

Pakdaman, M.; Sanaatiyan, M.M.; Ghahroudi, M.R.; , A line follower robot from design to implementation: Technical issues and problems, Computer and Automation Engineering (ICCAE), 2010 The 2nd International Conference on , vol.1, no., pp.5-9, 26-28 Feb. 2010
doi: 10.1109/ICCAE.2010.5451881
URL: http://ieeexplore.ieee.org/stamp/stamp.jsp?tp=&arnumber=5451881&isnumber=5451211

Pololu Baby Orangutan B Users Guide. Available from http://www.pololu.com/docs/pdf/0J14/baby_orangutan_b.pdf.

Figure 5 – Full Schematic of Line Following Robot

Appendix A

Figure 6 – Pololu Baby Orangutan B-328 Schematic diagram (Pololu Baby Orangutan B Users Guide)